## Trusted Platform Module (TPM)

The TPM is a dedicated microprocessor designed to offer facilities for generation of cryptographic keys, random number generation, remote attestation and data sealing. In this document, we use the TPM as the hardware root of trust of our Cloudproxy system.

To use the TPM we need to enable it, and take ownership of it. Taking ownership of a TPM is the process of setting up an authorization value (eg. password), which is subsequently required for certain kinds of TPM operations.

Following are instructions on how to setup TPM 1.2 and TPM 2.0 for Cloudproxy.

TPM 1.2

There are several keys in a TPM 1.2, which include:

TPM Endorsement Key (EK): This key is created by the manufacturer and cannot be removed. Sometimes it can be changed by the owner of the computer.

TPM Storage Root Key (SRK): Is the 2048 bit RSA key created when configuring the ownership. This key is stored inside the chip and can be removed. This key is also used to seal/unseal data.

TPM Attestation Identity Key (AIK): This key is used by the TPM to sign attestations (to the state of the PCRs). It is encrypted and integrity protected by the SRK.

Cloudproxy uses the SRK to seal/unseal data and the AIK to sign attestations. However, all we need to get Cloudproxy running is an AIK blob. Following are instructions on how to obtain it, and to provision the TPM for use by Cloudproxy.

- First, we install tools needed to talk to the TPM device.

go get -u -v github.com/google/go-tpm/…

- Reset your TPM device as follows.

- Disable the TPM device on your BIOS screen.
- Reboot the system by powering off, re-enable the TPM device in BIOS, and select the option to clear the device if you see one.

TPM devices require an assertion of physical presence to be cleared, and for most machines powering off and on does the trick. But ultimately how presence is asserted depends on your machine, so consult your BIOS manual for detailed instructions.
Also, note that the TPM device comes disabled by default.

- Take ownership of the device, which may need root privileges. Note that tpm-takeownership takes the new owner and SRK password from the environment, and that we do not set them. The current Cloudproxy implementation assumes these authorizations are not set, when talking to the TPM.

unset TPM_OWNER_AUTH
unset TPM_SRK_AUTH
sudo su --preserve-environment
tpm-takeownership

- Generate the aikblob and set permissions so that Cloudproxy may read it (note that this blob is encrypted and integrity protected by the TPM so it can be made public).

genaik
chmod 777 aikblob

Some troubleshooting tips can be found in the appendix.

TPM 2.0

Taking ownership of a TPM 2.0 is the process of setting up authorization values (eg. passwords) for the ownerAuth, endorsementAuth, and lockoutAuth types of authorization. Different TPM 2.0 commands require different types of authorization. A cleared TPM (using TPM2_Clear) has these values set to emptyAuth. To use Cloudproxy, we need to set each of the above authorization to emptyAuth (i.e. empty string as a password).

Enabling, taking ownership of and clearing a TPM 2.0 device is done entirely through BIOS. Consult your BIOS manual for detailed steps. Detailed instructions for the Intel NUC 5i5MYHE can be found in the appendix.

## tboot

Trusted Boot (tboot) is a pre-kernel/VMM module that uses Intel Trusted Execution Technology (Intel TXT) to perform a measured and verified launch of an OS kernel/VMM. It uses the TPM for this and when successfully run, it stores the kernel/VMM measurement in the TPM PCRs, which is then read by the root CloudProxy Tao.

Installation overview: After making sure your TPM device is enabled (see previous sections), download the tboot source tarball from sourceforge.net, and build and install using make and sudo make install. This build a tboot.gz image and places it in /boot for your bootloader to use when booting up. You will also need a SINIT AC module for your type of processor that you must manually place in /boot. This module enables TXT to load tboot. Finally you need to update your bootloader configuration to load tboot. Detailed instructions are in the appendix.

Some miscellaneous but probably useful notes on tboot follow.

txt-stat: The txt-stat tool installed by tboot is a Linux application that reads some of the TXT registers and will display the tboot boot log if tboot was run with 'logging=memory'. It is useful to run after a boot-up to check if tboot was run successfully.

Launch control policy (LCP) and Verified Launch Policy (VLP): These are policies that govern which measured launch environments (MLEs, for eg. tboot) and kernels TXT and tboot are allowed to launch respectively. Tboot comes with a set of tools to install LCPs and VLPs, however they are not needed for CloudProxy.

PCR values after trusted boot:

PCR 17 :
   It will be extended with the following values (in this order):
       -  The values as documented in the MLE Developers Manual
       -  SHA-1 hash of:  tboot policy control value (4 bytes) |
                          SHA-1 hash of tboot policy (20 bytes)
          : where the hash of the tboot policy will be 0s if TB_POLCTL_EXTEND_PCR17 is clear
PCR 18 :
   It will be extended with the following values (in this order):
       -  SHA-1 hash of tboot (as calculated by lcp_mlehash)
       -  SHA-1 hash of first module in grub.conf (e.g. Xen or Linux kernel)

## Appendix section: Troubleshooting TPM 1.2

tpm-takeownership fails: tpm-takeownership assumes the owner of the device is not set. Reset the TPM through BIOS as explained in the main TPM 1.2 section to reset ownership.

tpm-takeownership not found: Make sure $GOPATH/bin is in your PATH environment variable.

Using tpm-clear: is NOT recommended for resetting the device or clearing ownership, and it is known to leave the TPM in a mucked state from where tpm-takeownership does not work. Reset the TPM through BIOS as explained in the main TPM 1.2 section to reset the device/ownership.

Too many open handles?: Run tpm-keys to list and close all open handles.

## Appendix section: Installing tboot

To build tboot you first need to install trousers and libtspi-dev.

Installing TBOOT

- Download the source tarball from https://sourceforge.net/projects/tboot/

- In the untarred source directory run:
make

- In the same directory as above, run
sudo make install

- Check the /boot directory contains the tboot.gz file

- Download the SINIT AC module corresponding to your processor from
https://software.intel.com/en-us/articles/intel-trusted-execution-technology

- Copy the SINIT *.BIN file to your /boot directory

- To start using tboot, you need to update your bootloader configuration. If using grub2, run
sudo update-grub
And check that /boot/grub/grub.cfg now contains an entry for tboot which looks like:

menuentry 'Ubuntu GNU/Linux, with tboot 1.9.4 and Linux 4.4.0-24-generic' --class ubuntu --class gnu-linux --class gnu --class os --class tboot {

insmod multiboot2

insmod part_gpt

insmod ext2

set root='hd0,gpt2'

if [ x$feature_platform_search_hint = xy ]; then

search --no-floppy --fs-uuid --set=root --hint-bios=hd0,gpt2 --hint-efi=hd0,gpt2 --hint-baremetal=ahci0,gpt2  1df958d9-c01d-43d8-a1b2-18d022843d3f

else

search --no-floppy --fs-uuid --set=root 1df958d9-c01d-43d8-a1b2-18d022843d3f

fi

echo    'Loading tboot 1.9.4 ...'

multiboot2      /boot/tboot.gz logging=serial,memory

echo    'Loading Linux 4.4.0-24-generic ...'

module2 /boot/vmlinuz-4.4.0-24-generic root=UUID=1df958d9-c01d-43d8-a1b2-18d022843d3f ro quiet splash intel_iommu=on noefi

echo    'Loading initial ramdisk ...'

module2 /boot/initrd.img-4.4.0-24-generic

echo    'Loading sinit 5th_gen_i5_i7_SINIT_79.BIN ...'

module2 /boot/5th_gen_i5_i7_SINIT_79.BIN

}

- If using tboot with cloudproxy, you need to adjust your bootloader configuration differently (to add the initramfs). Instructions to do so can be found in a subsequent appendix.

More information about tboot can be found in the README in the tboot source directory.

## Appendix section: Troubleshooting TPM 2.0

TPM2 Error code 0x9a2:

This error code corresponds to ‘bad authorization’ and is returned when a command has incorrect authorization values. If you encounter this when using CloudProxy, it means your TPM is not provisioned properly, and you would need to clear the TPM.

TPM2 Error code 0x902:

This error code is returned when the TPM is out of memory for creating new objects. Run the TPM2_FlushContext command for all open handles (which can be obtained by running the TPM2_GetCapability command).

TPM2_Clear on the Intel NUC 5i5MYHE:

Clearing the TPM has the effect of setting all authorization values to emptyAuth (i.e. empty string as password). Clearing a TPM can be done through the maintenance screen of the BIOS, which is accessed by booting up the machine with the yellow BIOS jumper removed, and pressing the Esc key when the startup BIOS screen is shown.

## Appendix section: Setting up Cloudproxy enabled KVM host

In this section we will see how to set up a Cloudproxy enabled KVM host, which can launch independent Cloudproxy VMs (i.e. VMs with a stacked Cloudproxy host running)

The root Tao of our CloudProxy KVM host will be a “soft” Tao: i.e. a dummy Tao based on a keyset that is stored on disk, encrypted and integrity protected by a password.

The guest VMs in will be CoreOS instances.

- Generate soft Tao.
We set up the Tao domain directory and the host directory, where the domain and host configuration information will be stored. We copy in a domain template which we will use in this example and setup the soft Tao which generates a password protected keyset in the linux_tao_host directory.

mkdir /tmp/domain
cd /tmp/domain
mkdir linux_tao_host
cp $GOPATH/src/github.com/jlmucb/\
	cloudproxy/go/apps/simpleexample/SimpleDomain/\
	domain_template.simpleexample .
tao domain newsoft \
	-config_template domain_template.simpleexample \
	-soft_pass xxx ./linux_tao_host

- Initialize domain
We initialize the domain which will create policy keys, saved in the domain directory and protected by the password. It will also create a tao.config file which stores all the domain configuration information and is used by the host to load the domain.

tao domain init \
		-tao_domain . \
		-config_template domain_template.simpleexample \
		-pass xxx

- Create linux_host image
We statically build a linux host image (so that it is executable in the VM) and create a tarball of the linux_host binary and the host/domain configuration information it needs to run. When launching a VM, the KVM Cloudproxy unpacks this, mounts it on the VM and runs linux_host as a stacked Cloudproxy host.



CGO_ENABLED=0 go install -a \
	-installsuffix nocgo \
	 github.com/jlmucb/cloudproxy/go/apps/linux_host
cp $GOPATH/bin/linux_host .
tar -C . -czf linux_host.img.tgz $(ls .)

- Get CoreOS image
Get the latest stable image of CoreOS.

curl -G http://stable.release.core-os.net/amd64-usr/\
	current/coreos_production_qemu_image.img.bz2 > \
	coreos_production_qemu_image.img.bz2
bunzip2 coreos_production_qemu_image.img.bz2

- Setup ssh-agent
We generate a key pair and setup ssh-agent for SSHing into the VM instance.

ssh-keygen -t rsa -f ./id_rsa
ssh-add id_rsa

- Initialize and start host
We initialize and start the KVM Cloudproxy host

tao host init \
	-root \
	-tao_domain . \
	-pass xxx \
	-hosting kvm_coreos \
	-kvm_coreos_img coreos_production_qemu_image.img \
	-kvm_coreos_ssh_auth_keys id_rsa.pub

tao host start \
	- tao_domain . \
	- host ./linux_tao_host

- Launch hosted program (i.e. VM)
We launch a Cloudproxy VM through the Cloudproxy KVM host. We create another temporary directory where the linux_host tarball is unpacked and mounted on the VM and run from.


mkdir another_tmp
tao_launch run \
	-tao_domain . \
	-host ./linux_tao_host \
	-verbose kvm_coreos:linux_host.img.tgz \
	-- linux_host.img.tgz another_tmp 2222

## Appendix section: Building kernel and initrd image

- Download and unpack Linux kernel and busybox source
We use Busybox to provide a basic set of Unix utilities to use on the VM instance.

mkdir /tmp/tiny_linux

cd /tmp/tiny_linux

curl https://www.kernel.org/pub/linux/kernel/\
	v4.x/linux-4.0.3.tar.xz | tar xJf -
	curl https://busybox.net/downloads/busybox-1.23.2.tar.bz2 \

| tar xjf -

- Build busybox

mkdir -pv ../obj/busybox-x86
make O=../obj/busybox-x86 defconfig

make O=../obj/busybox-x86 menuconfig

The last command opens an interactive menu. We will select to build BusyBox statically to ensure it can run on the VM instance.

-> Busybox Settings
  -> Build Options
[ ] Build BusyBox as a static binary (no shared libs)

Select the above option, save your selection and quit the interactive menu.
Next we build Busybox as shown below.

cd ../obj/busybox-x86
	make -j2
	make install

- Build initrd image
Next we build the init RAM disk image.

mkdir -p /tmp/tiny_linux/initramfs/x86-busybox
cd /tmp/tiny_linux/initramfs/x86-busybox
mkdir -pv {bin,sbin,etc,proc,sys,usr/{bin,sbin}}
cp -av $TOP/obj/busybox-x86/_install/*

We will need an init script. The following minimal script works.

#!/bin/sh
mount -t proc none /proc
mount -t sysfs none /sys
mount -t devtmpfs -o mode=0755 udev /dev

exec /bin/sh

Save the above script in a file called init and make it executable.

chmod +x init

Finally we archive and compress this directory to create our initrd image.

find . -print0 \
    | cpio --null -ov --format=newc \
    | gzip -9 > /tmp/tiny_linux/obj/initramfs-busybox-x86.cpio.gz

- Build linux kernel.

cd /tmp/tiny_linux/linux-4.0.3
make O=../obj/linux-x86-basic x86_64_defconfig
make O=../obj/linux-x86-basic kvmconfig
make O=../obj/linux-x86-basic -j2

The kernel image can be found at /tmp/tiny_linux/obj/linux-x86-basic/arch/x86_64/boot/bzImage

## Appendix section: Launching VM with custom kernel through KVM
Cloudproxy

- Generate soft Tao.
We set up the Tao domain directory and the host directory, where the domain and host configuration information will be stored. We copy in a domain template which we will use in this example and setup the soft Tao which generates a password protected keyset in the linux_tao_host directory.

mkdir /tmp/domain
cd /tmp/domain
mkdir linux_tao_host
cp $GOPATH/src/github.com/jlmucb/\
	cloudproxy/go/apps/simpleexample/SimpleDomain/\
	domain_template.simpleexample .
tao domain newsoft \
	-config_template domain_template.simpleexample \
	-soft_pass xxx ./linux_tao_host

- Initialize domain
We initialize the domain which will create policy keys, saved in the domain directory and protected by the password. It will also create a tao.config file which stores all the domain configuration information and is used by the host to load the domain.

tao domain init \
		-tao_domain . \
		-config_template domain_template.simpleexample \
		-pass xxx

- Initialize and start host
We initialize and start the KVM Cloudproxy host

tao host init \
	-root \
	-tao_domain . \
	-pass xxx \
	-hosting kvm_custom \

tao host start	-tao_domain .

- Copy Kernel image and initrd image to domain directory
Replace the paths in the following lines with the path tyour kernel and initrd image.

cp /tmp/tiny_linux/obj/linux-x86-basic/arch/x86_64/boot/bzImage .
cp /tmp/tiny_linux/obj/initramfs-busybox-x86.cpio.gz .

- Launch hosted program (i.e. VM)
We launch a Cloudproxy VM through the Cloudproxy KVM host.

tao_launch run \
	-tao_domain . \
	-verbose kvm_custom: \
	-- bzImage initramfs-busybox-x86.cpio.gz 2222